Basopra Input Variables and Data

Basopra Model requires many input variables so it is important to know how to insert them in the files and what are the options available for each one of them. There are two input files

Input_Data

- Inverter_load_ratio

ILR is the DC capacity of the solar panels divided by the AC capacity of the inverter. This difference of capacities leads to some energy being lost or clipped during the sunniest hours of the year.

kWp/kW

- Curtailment

In some regions with substantial PV penetration, a feed-in  limit  is  set  above  which  PV  power  cannot  be  injected  to  the  grid  to  keep grid stability.

It is a % based on the nameplate capacity of the PV system. So 0.5 will be a maximum injection allowed of 50% of the PV power, if there is a 5 kWp PV system, it will be a maximum of 2.5 kW taht are allowed to be injected into the grid

- Inverter_efficiency

Self-explanatory

- Converter_efficiency

Self-explanatory

- Time_resolution

Time resolution of the input data (1 = 1 hour, 0.5 = 30 minutes, 0.25 = 15 minutes)

- number_of_years

Number of years to be simulated (must match input data)

- number_of_years

Number of days to be simulated (must match input data)

- Avoidance_PV_curtailment

Boolean to set if the strategy to avoid PV curtailment will be applied in the model (1 it will be applied, 0 it will be off)

- Demand_load_shifting

Boolean to set if the strategy of load shifting will be applied in the model (1 it will be applied, 0 it will be off)

- Demand_peak_shaving

Boolean to set if the strategy of peak shaving will be applied in the model (1 it will be applied, 0 it will be off)

- Technology

Battery Technology

Options:

NMC - Nickel Manganese Cobalt

NCA - Lithium Nickel Cobalt Aluminium

LFP - Lithium Iron Phosphate

LT0 - Lithium Titanate

ALA - Advanced Lead Acid

VRLA - Valve Regulated Lead Acid

- Capacity

Battery Capacity. Must be an integer number

kWh

- Capacity_tariff

Tariff that is additionally applied based on the capacity required by the installation. To work well it is important to verify if the Final Tariff calculation in the location is similar to the one applied in the model.

This value is defined in USD/kW/month (if the currency changes there is no problem, but must be Currency/kW/month)

- PV_norm

PV system size in kWp

- year_data

The year of the input data

- time_zone

Self-explanatory

Options:

US/Central

df_1h

- 164

In kWh. 164 is only the household id

- E_PV

Energy generated by the PV Panels (provided based on solar incidence)

- Price_flat

- Price_DT

- Export_price

Price paid by the energy provided to the generator when energy is returned to the grid

- Price_flat_mod

- Price_DT_mod

When a capacity price is introduced, a modification on the volumetric tariff ($/kWh) is made to guarantee that the households will not face a huge increment in the bill

Basopra Output Variables and Data

Two files are generated: df and aggregated_results.

df

Shows the optimum result identified for each iteration of the model. It is the file with the data that will be transferred to Danny's existing model in excel. Can be used to generate plots of the energy flows and visually compare different cases.

Most of the output variables and their units are identified in this table

aggregated_results

Shows aggregated results for the entire period defined in the input data file. Energy, Power and Financial data

- E_PV_batt

In the table

- E_PV_curt

In the table

- E_PV_grid

In the table

- E_PV_load

In the table

- E_char

In the table

- E_cons

Total energy consumed from the grid. It is a sum of the load that was supplied by the grid, and the load that was used to charge the battery in some situations. In the table is represented by Egrid.

- E_dis

In the table

- E_grid_batt

In the table

- E_grid_load

In the table

- E_loss_Batt

In the table

- E_loss_conv

In the table

- E_loss_inv

In the table

- E_loss_inv_PV

- E_loss_inv_batt

- E_loss_inv_grid

- E_demand

The energy load demanded by the system. Is represented as Eload in the table.

- E_PV

In the table

- App_comb

- SOC_mean

Mean battery state of charge

- SOC_max

Max battery state of charge

- SOC_min

Min battery state of charge

- DoD_mean

A battery’s depth of discharge (DoD) indicates the percentage of the battery that has been discharged relative to the overall capacity of the battery. For example, if you have a Tesla Powerwall that holds 13.5 kilowatt-hours (kWh) of electricity, and you discharge 13 kWh, the DoD is approximately 96 percent.

- DOD_max

Maximum DOD obtained in the cycles.

A value higher than 100 is possible if the battery charged more than once that day.

- DOD_min

Minimum DOD obtained in the cycles

- last_cap

- cap_fading

- last_SOH

- P_max_year_batt

- P_max_year_nbatt

- Capacity

- Tech

- PV_nom

- name

- results_PVbatt

- results_PV

- results

- EFC_nolifetime

Equivalent Full Cycles

To compute EFC, the discharge throughput is divided by the nominal capacity C of the battery.

- LS

Is the relation between how much energy was consumed from the grid vs. how much energy was demanded by the load when the tariff price was at the maximum. If there was no load shifting, this would result in 0, but with load shifting there will be less consumption from the grid during the maximum tariff price. The ideal result for this relation is 100%, what would mean that No energy was consumed from the grid during the maximum tariff period, it was all demanded from the battery.

- TSC

TSC - Percentage of PV energy driven to the Demand (Load) AND to the battery

- DSC

DSC - Percentage of PV energy driven to the Demand (Load)

- ISC

ISC - Percentage of PV energy driven to recharge the battery

- CU

CU - Percentage of PV energy curtailed

- PS_year

PS_year – it is a comparison between the power of the shaved peaks vs the original peaks (if there was no peak shaving).

- BS

BS - Relation between the total Battery discharged energy and the total demand (a comparison on how much energy the battery discharged vs what was required by the load)

- cycle_to_total

Related to the aging of the battery, to know which type of aging (calendric or due to cycling) has more weight.

- cases

We use different cases in the literature. Minimum, median and maximum values are used, but you can use the default one (cases==False)